AAE – Utopia Slough

5/14/2021

- Parked by the gate on Utopia Rd. and started survey at bridge

- Visibility poor (< 2 ft.) and channel deep (too deep for waders) and narrow

- Channel connected to furthest downstream area of Wiseman Creek on SLT property

- Very low but nonzero velocity

- One Chinook observed at bridge

- Finished survey at upstream end on SLT property where significant cover in channel but no visible fish

- Solinst level logger 2136247 with t-post and PVC housing installed at 16:03 downstream from the bridge on LB reed canary grass cove easily in line of sight from bridge but hidden by grass, gage at 0.73, 48.51542, -122.13810.

- And temperature logger 21063852 in solar shield wired to same housing, same lat/long.

- Loggers in a LB pocket behind some grass.

6/10/2021

- Boat reconnaissance

- Couldn’t get boat close enough to mouth to assess connectivity

6/23/2021

- Loggers present, string length 1.44 m, gage height 0.66 m.

8/18/2021

- Attempted snorkel survey but visibility terrible

- Habitat data collected only, no snorkel survey

- Possibly good spot to e-fish in September

- Water column appears to be very stratified, took measurements at the top and bottom

- Bottom DO readings may not be accurate because of suspended silt

- WPT 253 – DO logger installed on RBK in low hanging cluster of branches about 40 ft upstream from bridge

9/20/2021

- Checked conditions of site from the bridge to see if could e-fish later in the week

- Water looked clearer than last time

- Temp: 13.5°C / DO: 1.61 mg/L

9/21/2021

- Retrieved DO logger to move to site Z

9/22/2021

- Boat reconnaissance

- Can’t access outlet – in sketchy side channel with lots of large wood

10/6/2021

- Logger retrieval

- Removed level logger 3:45 pm, housing submerged, gage height 0.61 m

- Removed temperature logger 3:45 pm, pendant submerged

- Water still very turbid, no fish observed

- Connectivity unknown because on private property closer to outlet