AAF – Illabot Chum Channel

4/12/2021

- Walked from the upstream end (dead-end) of the slough to the culvert which has wire on the upstream end to exclude beaver

- Culvert wetted with unimpeded flow through it

- Took habitat measurements in areas with high fish concentrations and included numbers of some nearby fish in abundance estimate

- WPT 200 – adult spawner jaw (CO or CH?)

- WPT 207 – beaver screen at culvert under road

Pictures:

- IMG_1418 – Upper end of slough (dead-end)

- IMG_1420 – Culvert under powerline road with beaver exclusion wiring

- IMG_1461 – Night snorkeling

5/5/2021

- General – juvenile salmonids <100 mm seen in daytime, including at least mykiss and coho. Unclear whether this site will get disconnected; need to walk it at lower water, or anytime, to assess.

- Solinst level logger 2134270 with t-post and PVC housing gage installed at 11:21 in center channel halfway between the culvert under the road and the corner of the channel as it makes and L shaped turn from west to north: 48.49585 -121.49358. Water level near top of short housing. Photo looking north toward culvert.

- Onset pendant temperature logger 20772099 with PVC solar shield installed at 11:39 on the LB biggest clumpy willow between the level logger and the culvert, wired to several 3-6” stems in the shade off the edge of the submerged riprap with a rock weight: 48.49588 -121.49376 Photo looking west/southwest toward LB willow anchor, yellow circle around wire extending down:

6/22/2021

- All AAF loggers present (level, temp), level logger gage irrelevant because submerged… total housing height at top of markings was 0.90 m + 12 ¾ inches of water overtop. (Top of housing itself to water surface 6.4 inches.) Later conversion of string length only, 35 inches = string length 0.89 m.

8/15/2021

- Site is fully connected to Illabot Slough

10/7/2021

- Logger retrieval

- Retrieved level logger 11:00 am, submerged, gage height 0.84 m

- Retrieved temperature logger 11:01, submerged

- Water remains clear and cold

- Coho 80-100 mm observed

- Water flowing through culvert, site fully connected to Illabot Slough

10/14/2021

- Logger reinstallation (although shouldn’t have been reinstalled – needs to take out)

- Temperature logger reinstalled 10:30 am, logger (#21063848) new to site – moved from Minkler then first one in Illabot died due to a reset error

- Level logger reinstalled 10:30 am, gage height 0.9 m