Sauk LBK Slough (Site AY)

Site Map

- AY1 – site outlet

- WPT 135 – downstream end of small very shallow pool

Figure 1: Map of Site AY (site outside of Relative Elevation Model (REM) extent)

Temperature, Water Level, and Connectivity Trends

- No equipment installed at the site because does not appear to become disconnected

- Although the site was only visited once during the field season, it is unlikely that the site is able to hold enough water to sustain fish during low flows

- Given the high relative elevation of the site and steep outlet channel to the mainstem Sauk, it is likely that the site drains almost entirely when mainstem Sauk flows recede

6/11/2021

- Boat reconnaissance

- Site connected to mainstem Sauk in outlet channel with several inches of fast flowing water

- WPT 135 – downstream end of small very shallow (6 in deep) pool with several 30-50 mm salmonids

Figure 2. Looking upstream at small shallow pool holding several 30-50 mm salmonids