West of Stevens Creek (Site D)

Site Map (Figure 1)

- D1 – site outlet

- WPT 295 – habitat measurements taken at downstream end of slough 9/22/2021

- WPT 296 – habitat measurements taken closer to the upstream end of slough 9/22/2021

- Level logger – placed in deepest downstream location 6/10/2021 – 10/11/2021

- Temperature logger – placed in deepest downstream location 6/10/2021 – 10/11/2021

- Timelapse camera – placed upstream from D1 at vegetated berm/beaver dam 6/10/2021 – 10/11/2021

Temperature, Water Level, and Connectivity Trends

Figure 2. Changes in water level (m) and temperature (°C) in West of Stevens Creek site relative to Skagit discharge (cfs).  The gray horizontal bar shows estimated site disconnection ranging between a Skagit discharge of 7,500 – 9,000 cfs.

- Estimated Connectivity Threshold: 7,500 – 9,000 cfs (Skagit)

- Given the low relative elevation of the site to the Skagit mainstem, site connectivity is likely heavily influenced by mainstem discharge

- As Skagit flows recede the water level in the site appears to decline rapidly

- Figure 2 water level shows that site completely dried up but logger not in the very deepest spot so could have held water

- 6/10/2021 – 10/11/2021

  - Minimum Temperature:

  - Maximum Tempreature:

  - Average Tempereature:

6/10/2021

- Boat reconnaissance

- Site potentially disconnected with small amount of water flowing under vegetated berm/beaver dam that may not be fish passable (Figure 3)

- Subsurface flow coming out of ground just downstream of timelapse camera location

- Turbid water just upstream of vegetated berm/beaver dam so not able to snorkel (Figure 4)

- No fish observed likely because of poor visibility

Figure 3. Small trickle flowing under vegetated berm into outlet channel

Figure 4. Level logger just upstream of vegetated berm in turbid slough

9/22/2021

- Boat reconnaissance

- Site disconnected at large vegetated berm/beaver dam at timelapse camera location with no water flowing underneath (Figure 5)

- Outlet channel just downstream of vegetated berm/beaver dam dry for 10-15 ft

Figure 5. Site disconnected at vegetated berm with no flow underneath and downstream outlet channel partially dry

- Moderate turbidity in slough (Figure 6)

- A couple of juvenile salmonids observed surfacing

- WPT 295 – took temperature and DO measurements at downstream end of slough

  - Temperature: 13.0°C

  - DO: 1.43 mg/L

- WPT 296 – took temperature and DO measurements closer to the upstream end of slough

  - Temperature: 13.1°C

  - DO: 3.67 mg/L

  - Depth: 0.54 m

Figure 6. Looking upstream at slough from vegetated berm

10/11/2021

- Boat reconnaissance

- Site disconnected at large vegetated berm/beaver dam at timelapse camera location with no water flowing underneath

- Water turbid and cold

- No fish observed