L – Cumberland Oxbow

5/10/2021

- Accessed via Skagit Land Trust trail from S. Skagit Hwy

- General – From SLT trails, take first viewpoint option (it has a wood duck box and a decent sized alder overhead) on left then skirt pond heading downstream for a couple hundred feet to the first [mostly] pond-spanning tree, branchy, with loose bark.

- Solinst level logger 2136261 with t-post and PVC housing installed at 17:13 on LB side behind the tree and lily pads to shield it from view from the viewpoint, waypoint 101, 48.51549 -122.00660

- Also temperature logger 20772086 in solar shield wired around same log described above at same lat/long, circled in yellow:

5/11/2021

- Snorkel survey and time lapse camera installed

- Survey temp loggers died at the end of survey in House Slough so no longitudinal temperature recordings of Cumberland Oxbow

- Time lapse camera installed WPT 30 (48.514227, -122.007367) at downstream end of oxbow pond where narrows into two channels – camera in right channel (if facing toward Skagit) where narrows and dries with large wood fallen across channel

5/12/2021

- Boat reconnaissance

- No apparent outlets from mainstem – steep incline from mainstem water surface

- 48.512414, -122.008392 bank along Skagit near apparent outlet of Cumberland oxbow

- WPT 31 (48.510703, -122.00784) Cumberland Creek outlet – connected

6/21/2021

- All loggers present (level, temp), level logger submerged too deeply for checking string length.

- Barologger 2136290 installed at 48.51562 -121.00637, hanging just above eye level in the suckering 2” branches on the south side of an alder at the back of a clump of alders as the spur trail that runs down pond from the oxbow trail dead ends in blackberry detritus and trees.

- Yellow dots show access via main trail and water’s edge to Cumberland level and temp loggers (yellow circle). Yellow X shows new barologger installation.

8/16/2021

- Cumberland Creek consists of isolated pools at this time

- Attempted snorkel survey but visibility terrible and not able to snorkel

- Hundreds of 20-30 mm sticklebacks

- Thick vegetation throughout

- Western toads seen

- Took four habitat points with temps high and DO very low (DO may not be accurate because of silty bottom)

- No difference in temp/DO between top and bottom of water column

10/12/2021

- Logger retrieval

- Temperature logger retrieved 1:22 pm, submerged

- Level logger retrieved 1:27 pm, submerged

- Barologger retrieved 1:36 pm

- Site likely not connected